CEE 530  -  Prestressed Concrete

Exam Preparation Notes

Open-Book Reference  |  Hand Calculation Procedures  |  Project 1 Parameters

Student: Chidchanok Pleesudjai (Fen)

Arizona State University  |  Spring 2026

Instructor: Prof. Barzin Mobasher

SIGN CONVENTION: Compression = Positive (+) ¦ Tension = Negative (-) Origin: y = 0 at bottom of stems, upward positive. Eccentricity e = y_c - y_tendon (positive when tendon is BELOW centroid)

# 1   Section Properties

## 1.1  Shoelace Formula  (Polygon Vertices, CCW Order)

List vertices (x_i, y_i) counterclockwise. Close polygon by repeating vertex 1 at the end.

Area: A = (1/2) * ¦SUM(x_i * y_{i+1} - x_{i+1} * y_i)¦ Centroid (y from origin): y_c = SUM[(y_i + y_{i+1}) * (x_i*y_{i+1} - x_{i+1}*y_i)] / (6A) Centroidal moment of inertia: I_c = SUM[(y_i^2 + y_i*y_{i+1} + y_{i+1}^2) * (x_i*y_{i+1} - x_{i+1}*y_i)] / 12

## 1.2  Section Moduli and Kern Points

Distances from centroid to extreme fibers: y_t = y_max - y_c (centroid to top fiber) y_b = y_c - y_min (centroid to bottom fiber = y_c when y_min = 0) Section moduli: S_t = I_c / y_t (top fiber modulus, in^3) S_b = I_c / y_b (bottom fiber modulus, in^3) Kern points (no-tension zone boundaries): k_t = I_c / (A_c * y_t) = r^2 / y_t (kern toward top, in) k_b = I_c / (A_c * y_b) = r^2 / y_b (kern toward bottom, in) r^2 = I_c / A_c (radius of gyration squared)

Physical meaning of kern points: If the prestress resultant falls within the kern (k_b below centroid to k_t above), the entire cross-section stays in compression. Tendon eccentricity typically exceeds k_b for efficient design.

## 1.3  Project 1  (Double-T, y = 0 at stem soffit)

| Vertex coordinates (CCW): | (-60,26),(-33,26),(-32,0),(-28.25,0),(-27.25,26),(27.25,26),(28.25,0),(32,0),(33,26),(60,26),(60,28),(-60,28) |
| --- | --- |
| Area Ac | 487.0 in^2 |
| Moment of inertia Ic | 34,638.8 in^4 |
| Centroid yc (from bottom) | 20.362 in |
| Top fiber distance yt | 28.0 - 20.362 = 7.638 in |
| Bottom fiber distance yb | 20.362 in |
| Top modulus St | 34638.8 / 7.638 = 4,535.1 in^3 |
| Bottom modulus Sb | 34638.8 / 20.362 = 1,701.1 in^3 |
| Kern kt | 9.312 in |
| Kern kb | 3.493 in |
| Radius of gyration r^2 = Ic/Ac | 71.13 in^2 |

# 2   Stress Analysis Along the Beam

## 2.1  Loads and Moments

For a simply supported beam (moment at position x from left support):

M(x) = w * x * (L - x) / 2 (kip-in, x and L in inches, w in kip/in) M_max = w * L^2 / 8 (at midspan, x = L/2)

| Transfer moment Mi | Mi = w_sw * x * (L-x) / 2 [self-weight only] |
| --- | --- |
| Service moment MT | MT = (w_sw + w_SDL + w_LL) * x * (L-x) / 2 |
| --- Project 1 Loads --- |  |
| Self-weight w_sw | Ac * (150/1728) / 1000 = 0.04227 kip/in |
| SDL (2-in topping) w_SDL | 2 x 120 x (150/1728) / 1000 = 0.02083 kip/in |
| Live load w_LL | 420 lb/ft / 12 / 1000 = 0.03500 kip/in |
| Total service w_s | 0.09810 kip/in |
| Span L | (20+24+20) x 12 = 768 in (64 ft) |

## 2.2  Eccentricity of the Tendon Group  e_cg

When multiple tendons are at different heights, combine into one resultant F at eccentricity e_cg:

Force-weighted centroid eccentricity: e_cg(x) = SUM[ Pi * e_i(x) ] / SUM[ Pi ] where Pi = Aps_i * fpi_i Individual tendon eccentricity: e_i(x) = y_c - y_i(x) (positive when tendon is BELOW centroid) Harped tendon y-profile (trapezoidal): y_h(x) = y_sup + (y_drape - y_sup) * x / x_drape for 0 <= x <= x_drape y_h(x) = y_drape for x > x_drape NOTE: Use FORCE-weighted average (Pi = Aps*fpi), not area-weighted, when strand sizes or prestress levels differ.

| x (ft) | y_harped (in) | y_cg (in) | e_cg (in) |
| --- | --- | --- | --- |
| 0 ft | 20.362 (= yc) | (6+20.362)/2 = 13.181 | ('14.362 - 7.181 = 7.181', {'bold': True}) |
| 10 ft | 13.181 | (6+13.181)/2 = 9.590 | ('10.772', {'bold': True}) |
| 20 ft (drape pt) | 6.000 | (6+6)/2 = 6.000 | ('14.362', {'bold': True}) |
| 32 ft (midspan) | 6.000 | (6+6)/2 = 6.000 | ('14.362', {'bold': True}) |

## 2.3  Allowable Stresses  (CEE 530)

| Stage | Fiber | Limit Type | Formula | Project 1 Value |
| --- | --- | --- | --- | --- |
| Transfer | Top / Bottom | Compression | +0.60 f'ci | ('+2.880 ksi', {'bold': True}) |
| Transfer | Top / Bottom | Tension | -3*sqrt(f'ci psi)/1000 | ('-0.208 ksi', {'bold': True}) |
| Service (total) | Top | Compression | +0.60 f'c | ('+3.600 ksi', {'bold': True}) |
| Service (Class U) | Bottom | Tension | -6*sqrt(f'c psi)/1000 | ('-0.465 ksi', {'bold': True}) |

At Transfer: Use FULL force F (no losses). eta = 1.0 at transfer. At Service: Use EFFECTIVE force Fe = eta * F where eta = 1 - losses = 0.85 for Project 1.

## 2.4  Fiber Stress Equations

AT TRANSFER (eta = 1.0, F = Fi, M = Mi = Msw): f_top = +F/Ac - F*e/St + Mi/St (top fiber) f_bot = +F/Ac + F*e/Sb - Mi/Sb (bottom fiber) AT SERVICE (eta = 0.85, Fe = eta*F, M = MT = Msw+MSDL+MLL): f_top = +Fe/Ac - Fe*e/St + MT/St (top fiber) f_bot = +Fe/Ac + Fe*e/Sb - MT/Sb (bottom fiber) Term-by-term meaning: +F/Ac = uniform compression from prestress (always +) -F*e/St = top fiber TENSION from prestress moment (eccentric force) +F*e/Sb = bottom fiber COMPRESSION from prestress moment +M/St = top fiber COMPRESSION from applied moment -M/Sb = bottom fiber TENSION from applied moment

## 2.5  Step-by-Step Stress Check Procedure

Compute section properties (or look up): Ac, Ic, yc, St, Sb

Compute e_cg(x) from tendon profile using force-weighted centroid formula

Compute moments: Mi(x) = w_sw*x*(L-x)/2  and  MT(x) = w_s*x*(L-x)/2

Apply fiber stress equations at TRANSFER (eta=1, M=Mi) and SERVICE (eta=0.85, M=MT)

Compare f_top and f_bot against allowable limits. State PASS or FAIL.

Exam tip - Critical sections to check: x = 0 (support): M = 0, full eccentric force -> check transfer top tension and bottom compression x = L/2 (midspan): maximum moment -> check service bottom tension (usually governs) x = drape point (20 ft): transfer bottom compression may govern (large e, small M)

# 3   Ultimate Strength  (Mn)

## 3.1  beta_1  -  Compression Block Factor  (ACI 318-19 Table 22.2.2.4.3)

beta_1 = 0.85 f'c <= 4,000 psi beta_1 = 0.85 - 0.05*(f'c - 4000)/1000 >= 0.65 4,000 < f'c <= 8,000 psi beta_1 = 0.65 f'c > 8,000 psi Project 1: f'c = 6,000 psi beta_1 = 0.85 - 0.05*(6000-4000)/1000 = 0.85 - 0.10 = 0.75

## 3.2  gamma_p  -  Bonded Tendon Factor  (ACI 318-19 Table 22.3.2.1)

| fpy/fpu | gamma_p (ACI 318-19) |
| --- | --- |
| >= 0.9 | 0.28 |
| >= 0.8 and < 0.9 | 0.40 |
| < 0.8 | 0.55 |

Project 1:  fpy/fpu = 243/270 = 0.90  -->  gamma_p = 0.28

## 3.3  Prerequisites Before Computing fps

Check fse >= 0.5*fpu:  fse = eta * fpi = 0.85 * 163.4 = 138.9 ksi  >=  0.5*270 = 135 ksi  [OK]

dp = depth to tendon group from COMPRESSION face (top for positive moment)

Project 1 midspan: dp = h - (yc - e_cg) = 28 - (20.362 - 14.362) = 28 - 6.000 = 22.000 in

STOP: dp is measured from the COMPRESSION face (TOP for simply-supported beam under positive moment), NOT from the bottom. dp = h - y_c + e_cg = section height - (centroid height) + eccentricity = 28 - 20.362 + 14.362 = 22.000 in

## 3.4  Stress in Prestressing Steel at Nominal Strength

ACI 318-19 Eq. 22.3.2.1 (bonded tendons, no mild steel in tension zone):

fps = fpu * [1 - (gamma_p/beta_1) * rho_p * fpu/f'c] where: rho_p = Aps / (b_eff * dp) b_eff = effective flange width for T-section (= 120 in for Project 1) dp = depth to tendon group from compression face (in) General form with mild steel: fps = fpu*[1-(gamma_p/beta_1)*(rho_p*fpu/f'c + (d/dp)*omega - (d'/dp)*omega')] where omega = rho*fy/f'c (tension), omega' = rho'*fy/f'c (compression)

## 3.5  Depth of Equivalent Stress Block  a

Rectangular section assumption (check a <= hf first): a = Aps * fps / (0.85 * f'c * b_eff) c = a / beta_1 (neutral axis depth from top) T-section check: If a <= hf (flange thickness): rectangular formula is valid [OK] If a > hf: use T-section formula (see below) T-section formula (when a > hf): Ff = 0.85*f'c*(b_eff-b_w)*hf (flange overhang compression force) Aps_w = Aps - Ff/fps (equivalent Aps for web only) a_w = Aps_w*fps / (0.85*f'c*b_w) (stress block depth in web) Mn = Aps_w*fps*(dp-a_w/2) + Ff*(dp-hf/2)

## 3.6  Ductility Check  (epsilon_t)

epsilon_t = [(dp - c) / c] * 0.003 (net tensile strain at nominal strength) epsilon_t >= 0.005: phi = 0.90 (tension-controlled, preferred) epsilon_t = 0.004: phi = 0.81 (ACI 9.3.3, minimum allowed for flexure) epsilon_t < 0.004: Section is compression-controlled -- redesign required

## 3.7  Factored Moment  Mu  and Strength Check

Mu = 1.2*MD + 1.6*ML (ACI 318-19 Eq. 5.3.1b) phi*Mn >= Mu [Strength requirement] phi*Mn >= 1.2*Mcr [Minimum reinforcement ACI 318-19 Sec 24.3.2] Modulus of rupture and cracking moment: fr = 7.5*sqrt(f'c psi) / 1000 (ksi, ACI 318-19 Sec 19.2.3) Mcr = fr * Sb (bottom fiber cracks first under positive moment)

## 3.8  Project 1 -- Ultimate Check Summary (midspan, 4 strands)

| Total Aps | 4 x 0.153 = 0.612 in^2 |
| --- | --- |
| dp (from top, = h - yc + e_cg) | 28 - 20.362 + 14.362 = 22.000 in |
| rho_p = Aps/(b_eff*dp) | 0.612 / (120 x 22.000) = 0.000232 |
| gamma_p | 0.28 (fpy/fpu = 0.90 >= 0.9) |
| beta_1 (f'c = 6.0 ksi) | 0.75 |
| fps | 270*[1-(0.28/0.75)*0.000232*270/6.0] = 270*(1-0.00462) = 268.75 ksi |
| a (check vs hf = 2 in) | 0.612*268.75/(0.85*6.0*120) = 0.255 in < 2 in [Rectangular OK] |
| c = a/beta_1 | 0.255/0.75 = 0.340 in |
| Mn | 0.612*268.75*(22.000-0.255/2) = 0.612*268.75*21.873 = 3,596 kip-in |
| epsilon_t | [(22.000-0.340)/0.340]*0.003 = 191.6*0.003 = 0.575 >> 0.005 [Tension-controlled] |
| phi | 0.90 (tension-controlled) |
| phi*Mn | 0.90 x 3,596 = 3,237 kip-in = 269.8 kip-ft |
| fr = 7.5*sqrt(6000)/1000 | 0.581 ksi |
| Mcr = fr * Sb | 0.581 x 1701.1 = 988 kip-in |
| 1.2*Mcr | 1.2 x 988 = 1,186 kip-in |
| phi*Mn vs 1.2*Mcr | 3,237 >> 1,186 [Min. reinforcement OK] |

# 4   Feasibility Design Chart  (Magnel Diagram)

## 4.1  Purpose

The Magnel diagram shows all combinations of (1/F, e) that satisfy all four stress constraints simultaneously. The feasible zone is the intersection of all four regions. F is the TOTAL prestress force at the section, e = e_cg(x) is the force-weighted eccentricity.

## 4.2  Moments at Each Analysis Section

Mi(x) = w_sw * x * (L-x) / 2 [transfer: self-weight only] MT(x) = (w_sw + w_SDL + w_LL) * x * (L-x) / 2 [service: all loads]

## 4.3  The Four Magnel Lines  (Naaman Numbering)

Starting from the stress inequalities and rearranging to isolate 1/F:

Define denominators (all positive for typical beams): dI = fti + Mi/St fti = ¦tension allowable¦ at transfer (+0.208 ksi) dII = fci + Mi/Sb fci = compression allowable at transfer (+2.880 ksi) dIII = MT/Sb - fts fts = ¦tension allowable¦ at service (+0.465 ksi) dIV = fcs - MT/St fcs = compression allowable at service (+3.600 ksi) NOTE: dIII must be > 0 for upper bound to exist. If MT = 0 (at support), dIII <= 0 and Line III gives no constraint -> F_min = 0 at that section.

| Line | Condition | Inequality | Bound type | Governs |
| --- | --- | --- | --- | --- |
| I | Top @ Transfer, Tension >= -fti | 1/F <= (e/St - 1/Ac) / dI [if dI>0, upper bound] | UPPER bound on 1/F | Near support |
| II | Bot @ Transfer, Compr. <= +fci | 1/F >= (1/Ac + e/Sb) / dII | LOWER bound on 1/F | Drape point |
| III | Bot @ Service, Tension >= -fts | 1/F <= eta*(1/Ac + e/Sb) / dIII | ('UPPER bound on 1/F', {'bold': True}) | ('Midspan (governs F_min)', {'bold': True}) |
| IV | Top @ Service, Compr. <= +fcs | 1/F >= eta*(1/Ac - e/St) / dIV | LOWER bound on 1/F | Midspan |

Feasible zone: max(Line II, Line IV) <= 1/F <= min(Line I, Line III) AND 1/F > 0 AND e_geo_min <= e <= e_geo_max where e_geo_max = yb - cover PITFALL: If denominator dI or dIV goes negative as e varies, the inequality FLIPS. Always check sign.

## 4.4  Finding F_min at a Given Section and Eccentricity

Line III sets the UPPER bound on 1/F. Larger 1/F = smaller F. (1/F)_max = eta * (1/Ac + e/Sb) / dIII Minimum required prestress force: F_min = 1 / (1/F)_max = dIII / [eta * (1/Ac + e/Sb)] Maximum allowed prestress force (from Line II): F_max = dII / (1/Ac + e/Sb)

## 4.5  Governing Section and Number of Strands

Compute Mi(x) and MT(x) at all analysis sections

Compute e_cg(x) at each section from the tendon profile

Compute dIII at each section.  If dIII <= 0: F_min = 0 at that section

Compute F_min at each section.  The LARGEST F_min governs (usually midspan)

Number of strands: n = ceil(F_min / Pi) where Pi = Aps * fpi per strand

Check F_provided = n * Pi against F_max at ALL sections -- must not exceed upper bound anywhere

## 4.6  Project 1 -- Feasibility Results

| Section x | Mi (kip-in) | MT (kip-in) | e_cg (in) | F_min (kip) | F_max (kip) |
| --- | --- | --- | --- | --- | --- |
| 0 ft | 0 | 0 | 7.181 | ('0', {'bold': True}) | ('459', {'bold': True}) |
| 10 ft | 1,643.6 | 3,814.4 | 10.772 | ('249', {'bold': True}) | ('459', {'bold': True}) |
| 20 ft (drape) | 2,678.5 | 6,216.1 | 14.362 | ('357', {'bold': True}) | ('424.4', {'bold': True}) |
| 32 ft (midspan) | 3,116.8 | 7,233.3 | 14.362 | ('424.5', {'bold': True}) | ('449', {'bold': True}) |

Governing: x = 32 ft (midspan) F_min = 424.5 kip n = ceil(424.5 / 25.0) = ceil(16.98) = 17 strands F_provided = 17 x 25.0 = 425 kip Note on F_max conflict: F_max at x=20 ft = 424.4 kip < F_min at x=32 ft = 424.5 kip Gap = 0.1 kip (0.2% overage). For engineering practice: 17 strands is the only choice.

KEY PITFALLS in Feasibility Problems: 1. If dIII <= 0: no service tension upper bound -> F_min = 0 at that section (x=0, MT=0) 2. Line III gives UPPER bound on 1/F = LOWER bound on F. Lower 1/F = higher F = stronger beam. 3. Line II gives LOWER bound on 1/F = UPPER bound on F. Check F <= F_max at ALL sections. 4. At transfer use full F (no losses). At service use Fe = eta*F. 5. e_cg uses FORCE-weighted average (Pi = Aps*fpi), not area average, when strand sizes differ. 6. Denominator formulas use MAGNITUDE of allowable stresses (treat fti and fts as positive).

# Appendix  -  Quick Reference Card  (Project 1 Numbers)

## A.1  Material Properties

| Property | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Concrete 28-day strength | f'c | 6.0 | ksi |
| Concrete strength at transfer | f'ci | 4.8 | ksi |
| Strand ultimate strength | fpu | 270 | ksi |
| Strand yield strength | fpy = 0.90*fpu | 243 | ksi |
| Strand initial prestress | fpi = Pi/Aps | 163.4 | ksi |
| Strand area (0.5-in dia.) | Aps | 0.153 | in^2 |
| Initial force per strand | Pi = Aps*fpi | 25.0 | kip |
| Prestress losses | losses | 15% | fraction 0.15 |
| Loss factor | eta = 1-losses | 0.85 | dimensionless |
| Effective force/strand | Pe = eta*Pi | 21.25 | kip |
| Number of strands | n (design) | 17 | strands |
| Total initial force | F = n*Pi | 425 | kip |
| Total effective force | Fe = eta*F | 361.25 | kip |

## A.2  Allowable Stresses  (CEE 530 Class U)

| Condition | Limit Formula | Project 1 Value |
| --- | --- | --- |
| Transfer -- Compression | +0.60 * f'ci | +2.880 ksi |
| Transfer -- Tension | -3*sqrt(f'ci psi)/1000 | -0.208 ksi |
| Service Total -- Compr. | +0.60 * f'c | +3.600 ksi |
| Service -- Tension (U) | -6*sqrt(f'c psi)/1000 | -0.465 ksi |

## A.3  Section Properties  (Double-T)

| Property | Value | Unit |
| --- | --- | --- |
| Ac | 487.0 | in^2 |
| Ic | 34,638.8 | in^4 |
| yc (centroid from bottom) | 20.362 | in |
| yt (centroid to top fiber) | 7.638 | in |
| yb (= yc, centroid to bot) | 20.362 | in |
| St (top section modulus) | 4,535.1 | in^3 |
| Sb (bottom section modulus) | 1,701.1 | in^3 |
| kt (kern toward top) | 9.312 | in |
| kb (kern toward bottom) | 3.493 | in |
| r^2 = Ic/Ac | 71.13 | in^2 |

## A.4  Tendon Layout  (Project 1, 4 strands at midspan)

| Tendon | Type | y at support | y at drape (x=240in) | e at midspan |
| --- | --- | --- | --- | --- |
| 1, 3 (outer) | Straight | 6.00 in | 6.00 in | 14.362 in |
| 2, 4 (inner) | Harped | 20.362 in (= yc) | 6.00 in | 14.362 in |