Appendix A

Part 3B — Detailed Calculations at Section 2: x = 120 in. (10.0 ft)

Effective Depth dp

Tendon centroid at x = 120 in.:

T1,T3 (straight): y = 6.00 in.

T2,T4 (harped):   y = 20.36 + (6.00−20.36)×(120/240) = 13.18 in.

y_ps = (6.00 + 13.18 + 6.00 + 13.18) / 4 = 9.590 in.

e = yc − y_ps = 20.362 − 9.590 = 10.772 in.

dp = max(h − y_ps, 0.80h) = max(28.00 − 9.590, 22.40) = max(18.41, 22.40)

dp = 22.40 in.   [0.80h governs]

Prestress & Dead-Load Stresses

fce = Pe/Ac + Pe × e × yb / Ic

= 85.0/487.0 + 85.0 × 10.772 × 20.362 / 34,638.82

= 0.1745 + 0.5383

fce = 0.7128 ksi

Md = 0.06311 × 120 × (768−120) / 2 = 0.06311 × 120 × 648/2

Md = 2,453.0 kip-in.

fd = 2,453.0 × 20.362 / 34,638.82

fd = 1.4423 ksi

Cracking Moment Mcr

fr + fce − fd = 0.4648 + 0.7128 − 1.4423 = −0.2648 ksi  < 0  ✖

Mcr = 0   (DL tension exceeds prestress + rupture)

Flexural-Shear Strength Vci

Vi   = 0.08100 × (384 − 120) = 0.08100 × 264 = 21.384 kip

Mmax = 0.08100 × 120 × 648/2 = 3,149.3 kip-in.

Vd   = 0.06311 × 264 = 16.661 kip

Term 1: 0.6 × 0.07746 × 7.50 × 22.40 = 7.808 kip

Term 2: Vd = 16.661 kip

Term 3: (Vi/Mmax) × Mcr = 0  (Mcr = 0)

Sum:    7.808 + 16.661 + 0 = 24.47 kip

Bounds check:

Vci,min (ACI floor) = 22.12 kip    Vci,max (CEE cap) = 65.07 kip

Floor:  24.47 kip  ≥  22.12 kip  →  PASS

Cap:    24.47 kip  ≤  65.07 kip  →  PASS

Vci = 24.47 kip   (no bound governs; Mcr = 0 limits the value)

Web-Shear Strength Vcw

fpc = 0.1745 ksi     Vp = 2.538 kip (x = 120 in., sloped region)

Vcw = (3.5×0.07746 + 0.3×0.1745) × 7.50 × 22.40 + 2.538

= 54.343 + 2.538

Vcw = 56.881 kip

Vc, Vu, Vs, Stirrup

Vc = min(Vci, Vcw) = min(24.47, 56.881)

Vc = 24.47 kip   → Vci GOVERNS (flexural-shear controls)

φVc = 0.75 × 24.47 = 18.35 kip

Vu = 0.13173 × (384 − 120) = 0.13173 × 264

Vu = 34.78 kip

Vu = 34.78 kip  >  φVc = 18.35 kip  →  FAIL

STATUS: Shear reinforcement required

Vs,req = max(34.78/0.75 − 24.47, 0) = max(46.37 − 24.47, 0)

Vs,req = 21.90 kip

Vs,req = 21.90 kip  ≤  Vs,max = 104.1 kip  →  PASS

Av/s_demand = 21.90 / (60 × 22.40) = 21.90/1344

Av/s_demand = 0.01629 in.²/in.   > Av/s_min = 0.00726  →  demand governs

s_req = 0.22 / 0.01629 = 13.50 in.

s_max = 21.0 in.   [Vs = 21.90 < 4√f′c×bw×dp = 52.06 → basic limit]

s = min(13.50, 21.0) = 13 in.   ← USE

Verify capacity:

Vs,prov = 0.22 × 60 × 22.40 / 13 = 22.74 kip

φ(Vc + Vs) = 0.75 × (24.47 + 22.74) = 0.75 × 47.21 = 35.41 kip

φ(Vc + Vs) = 35.41 kip  ≥  Vu = 34.78 kip  →  PASS

Part 3C — Detailed Calculations at Section 3: x = 240 in. (20.0 ft)

Effective Depth dp

Tendon centroid at x = 240 in. (drape point):

T1,T3 (straight): y = 6.00 in.

T2,T4 (harped):   y = 6.00 in. (at drape point)

y_ps = (6.00 + 6.00 + 6.00 + 6.00) / 4 = 6.000 in.

e = 20.362 − 6.000 = 14.362 in.

dp = max(28.00 − 6.00, 22.40) = max(22.00, 22.40)

dp = 22.40 in.   [0.80h governs]

Prestress & Dead-Load Stresses

fce = 85.0/487.0 + 85.0 × 14.362 × 20.362 / 34,638.82

= 0.1745 + 0.7177

fce = 0.8922 ksi

Md = 0.06311 × 240 × 528/2

Md = 3,997.7 kip-in.

fd = 3,997.7 × 20.362 / 34,638.82

fd = 2.3505 ksi

Cracking Moment Mcr

fr + fce − fd = 0.4648 + 0.8922 − 2.3505 = −0.9936 ksi  < 0  ✖

Mcr = 0   (DL tension exceeds prestress + rupture)

Flexural-Shear Strength Vci

Vi   = 0.08100 × (384 − 240) = 0.08100 × 144 = 11.664 kip

Mmax = 0.08100 × 240 × 528/2 = 5,132.2 kip-in.

Vd   = 0.06311 × 144 = 9.088 kip

Term 1: 7.808 kip     Term 2: 9.088 kip     Term 3: 0 (Mcr = 0)

Sum:    7.808 + 9.088 + 0 = 16.90 kip

Bounds check:

Vci,min (ACI floor) = 22.12 kip    Vci,max (CEE cap) = 65.07 kip

Floor:  16.90 kip  <  22.12 kip  →  raised to Vci,min  →  FAIL

Vci = max(16.90, 22.12) = 22.12 kip   (ACI floor governs)

Web-Shear Strength Vcw

fpc = 0.1745 ksi     Vp = 0 kip (x = 240 in., flat zone)

Vcw = (3.5×0.07746 + 0.3×0.1745) × 7.50 × 22.40 + 0

Vcw = 54.343 kip

Vc, Vu, Vs, Stirrup

Vc = min(22.12, 54.343)

Vc = 22.12 kip   → Vci GOVERNS (ACI floor controls)

φVc = 0.75 × 22.12 = 16.59 kip

Vu = 0.13173 × (384 − 240) = 0.13173 × 144

Vu = 18.97 kip

Vu = 18.97 kip  >  φVc = 16.59 kip  →  FAIL

STATUS: Shear reinforcement required

Vs,req = max(18.97/0.75 − 22.12, 0) = max(25.29 − 22.12, 0)

Vs,req = 3.17 kip

Av/s_demand = 3.17/1344 = 0.00236 in.²/in.

Av/s_min = 0.00726  >  0.00236  →  minimum governs

s_req = 0.22 / 0.00726 = 30.30 in.    s_max = 21.0 in.

s = min(30.30, 21.0) = 21 in.   ← USE

Verify capacity:

Vs,prov = 0.22 × 60 × 22.40 / 21 = 14.08 kip

φ(Vc + Vs) = 0.75 × (22.12 + 14.08) = 0.75 × 36.20 = 27.15 kip

φ(Vc + Vs) = 27.15 kip  ≥  Vu = 18.97 kip  →  PASS

Part 3D — Detailed Calculations at Section 4: x = 384 in. (32.0 ft, midspan)

Effective Depth dp

Tendon centroid at x = 384 in. (midspan, flat zone):

All tendons at y = 6.00 in.   →  y_ps = 6.000 in.

e = 20.362 − 6.000 = 14.362 in.

dp = max(22.00, 22.40)

dp = 22.40 in.   [0.80h governs]

Prestress & Dead-Load Stresses

fce = 85.0/487.0 + 85.0 × 14.362 × 20.362 / 34,638.82

fce = 0.8922 ksi   (same as Section 3)

Md = 0.06311 × 384 × 384/2

Md = 4,653.2 kip-in.

fd = 4,653.2 × 20.362 / 34,638.82

fd = 2.7351 ksi

Cracking Moment Mcr

fr + fce − fd = 0.4648 + 0.8922 − 2.7351 = −1.3782 ksi  < 0  ✖

Mcr = 0   (DL tension exceeds prestress + rupture)

Flexural-Shear Strength Vci

Vi   = 0.08100 × (384 − 384) = 0 kip   (midspan)

Vd   = 0.06311 × (384 − 384) = 0 kip   (midspan)

Term 1: 7.808 kip     Term 2: 0 kip     Term 3: 0 (Mcr = 0)

Sum:    7.808 kip

Bounds check:

Vci,min (ACI floor) = 22.12 kip    Vci,max (CEE cap) = 65.07 kip

Floor:  7.808 kip  <  22.12 kip  →  raised to Vci,min  →  FAIL

Vci = max(7.808, 22.12) = 22.12 kip   (ACI floor governs)

Web-Shear Strength Vcw

fpc = 0.1745 ksi     Vp = 0 kip (flat zone)

Vcw = 54.343 kip   (same as Section 3)

Vc, Vu, Vs, Stirrup

Vc = min(22.12, 54.343)

Vc = 22.12 kip   → Vci GOVERNS (ACI floor controls)

φVc = 0.75 × 22.12 = 16.59 kip

Vu = 0 kip   (midspan of simply supported beam)

Vu = 0 kip  ≤  φVc = 16.59 kip  →  PASS

STATUS: Only minimum stirrups required

Vs,req = 0 kip

Av/s_min = 0.00726 governs    s = min(0.22/0.00726, 21.0) = min(30.30, 21.0)

s = 21 in.   ← USE