Project 1 — Precast Prestressed Beam

Assignment 4: Shear Design

CEE 530 Prestressed Concrete

Spring 2026

Due: 3/23/2026

Group 3

Hailey Crosson

Chidchanok Pleesudjai (Fen)

1. Given Data

1.1 Section Properties

| Property | Value |
| --- | --- |
| h | 28.00 in. |
| Ac | 487.000 in² |
| Ic | 34,638.8 in⁴ |
| yc (from bottom) | 20.362 in. |
| yb | 20.362 in. |
| yt | 7.638 in. |
| bw (sum of stem widths) | 7.50 in. |
| St = Ic/yt | 4,535.1 in³ |
| Sb = Ic/yb | 1,701.1 in³ |

1.2 Material Properties

| Property | Value |
| --- | --- |
| f′c | 6.0 ksi |
| f′ci | 4.8 ksi |
| Ec | 4,700 ksi |
| fpu | 270 ksi |
| fpy | 243 ksi |
| Eps | 28,500 ksi |
| fy (stirrups) | 60 ksi |
| λ | 1.0 (normal weight) |
| φv | 0.75 |

1.3 Prestress Data

Four 0.5-in. diameter Grade 270 strands, each with Aps = 0.153 in². Initial force per strand Pi = 25.0 kip; total initial prestress Fi = 100.0 kip. With 15% losses (η = 0.85), effective prestress Pe = 85.0 kip.

Tendons 1 and 3 are straight at y = 6.00 in. Tendons 2 and 4 are trapezoidal harped: y = 20.36 in. at supports, sloping to y = 6.00 in. at x = 240 in., flat to x = 528 in., then sloping back to y = 20.36 in.

The vertical component of the effective prestress force, Vp = Pe sin(α), where α = arctan(Δy/Δx) is the tendon inclination angle. For harped tendons 2 and 4 in the sloped region (0 ≤ x ≤ 240 in.):

tan(α) = (20.36 − 6.00) / 240 = 14.36 / 240 = 0.05983

Vp = 2 tendons × 21.25 kip × sin(α) = 2 × 21.25 × 0.05972 = 2.538 kip  (upward)

In the flat region (240 ≤ x ≤ 528 in.), the tendon slope is zero and Vp = 0. Thus Vp = 2.538 kip at Sections 1 and 2, and Vp = 0 at Sections 3 and 4.

1.4 Applied Loads

| Load | kip/in | kip/ft |
| --- | --- | --- |
| wsw (self-weight) | 0.04227 | 0.5073 |
| wSDL (2-in. topping) | 0.02083 | 0.2500 |
| wLL (live load) | 0.03500 | 0.4200 |
| wDL = wsw + wSDL | 0.06311 | 0.7573 |
| wu = 1.2·wDL + 1.6·wLL | 0.13173 | 1.5808 |

2. Shear Design Method (CEE 530 / ACI 318-19)

ACI 318-19 requires that the factored shear demand Vu at each section shall not exceed the design shear capacity φVn, where Vn = Vc + Vs. The concrete contribution Vc is taken as the lesser of two failure modes: flexural-shear cracking (Vci) and web-shear cracking (Vcw).

Flexural-shear cracking (Vci) initiates as a flexural crack that subsequently turns into a diagonal shear crack. It governs in regions of high moment (near midspan). The computation requires the cracking moment Mcr, the unfactored dead-load shear Vd, and the factored external-load shear/moment pair Vi/Mmax. A lower bound Vci,min is enforced to prevent unconservative results when Mcr is small or zero.

Web-shear cracking (Vcw) initiates as a diagonal crack at the neutral axis of the beam before any flexural cracking occurs. It governs near the supports where shear is high and moment is low. The calculation depends on the compressive stress at the centroid fpc and the vertical component of the prestress force Vp (which acts upward and helps resist applied shear).

If Vu > φVc, transverse reinforcement (stirrups) shall be provided to carry the excess shear Vs = Vu/φ − Vc. Even when Vu ≤ φVc, ACI 318-19 requires a minimum amount of shear reinforcement (Av/s)
      min per Table 9.6.3.3.

The CEE 530 course convention uses Vci,min = 5×λ×√f′c×bw×dp (instead of the ACI 318-19 code value of 1.7).

2.1 Key Equations

Flexural-shear strength (ACI 318-19 Eq. 22.5.8.3.1a):

Eq. (1): Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr ≥ Vci,min = 5×λ×√f′c×bw×dp

where Vd = shear from unfactored dead load (SW + SDL); Vi and Mmax = factored shear and moment from superimposed loads only (1.2×wSDL + 1.6×wLL).

Cracking moment (ACI 318-19 Eq. 22.5.8.3.2a):

Eq. (2): Mcr = (Ic/yb)(6×λ×√f′c + fce − fd)

where fce = compressive stress at the bottom fiber due to effective prestress alone (fce = Pe/Ac + Pe×e×yb/Ic); fd = tensile stress at the bottom fiber due to unfactored dead-load moment (fd = Md×yb/Ic). When fd > 6λ√f′c + fce, the section is already cracked under dead load and Mcr = 0; in that case Vci,min governs.

Web-shear strength (ACI 318-19 Eq. 22.5.8.3.2):

Eq. (3): Vcw = (3.5×λ×√f′c + 0.3×fpc)×bw×dp + Vp

where fpc = compressive stress at the centroid of the section due to effective prestress (fpc = Pe/Ac); Vp = vertical component of the effective prestress force from inclined (harped) tendons.

Concrete shear capacity:

Eq. (4): Vc = min(Vci, Vcw)

Required steel shear contribution (when Vu > φVc):

Eq. (5): Vs,req = max(Vu/φv − Vc, 0)

Stirrup spacing for vertical stirrups (αs = 90°):

Eq. (6): Av/s = Vs/(fy×dp)

where Av = total cross-sectional area of the stirrup legs; s = stirrup spacing; fy = yield strength of stirrup steel; dp = effective depth to the prestress centroid.

2.2 Effective Depth and Critical Section

Per ACI 318-19 Section 22.5.3.2, the effective depth for shear shall be taken as:

dp = max(h − yps, 0.80×h) = max(28.00 − yps, 22.40) in.

Since the tendon centroid yps is relatively high in the section, 0.80×h = 22.40 in. governs at all sections. The critical section for shear is located at dp = 22.40 in. (1.87 ft) from the face of the support (ACI 318-19 R22.5.3.2).

3. Detailed Calculations at Design Sections

3.1 Section 1: x = 18 in. (1.5 ft)

Prestress at section

yps = 12.642 in.,   e = 7.721 in.

Cracking Moment

fce = Pe/Ac + Pe×e×yb/Ic

= 85.0/487.0 + 85.0×7.721×20.362/34,638.8

= 0.1745 + 0.3858 = 0.5603 ksi

fd = Md×yb/Ic

where Md = wDL×x×(L−x)/2 = 0.06311×18×(768−18)/2 = 426.0 kip-in

fd = 426.0×20.362/34,638.8 = 0.2504 ksi

fr + fce − fd = 0.4648 + 0.5603 − 0.2504 = 0.7747 ksi > 0

Mcr = (Ic/yb)×0.7747 = 1701.1×0.7747 = 1,317.8 kip-in

Flexural-Shear Strength (Vci)

From Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

Factored external-load shear and moment (superimposed loads only, excluding self-weight):

wext = 1.2×wSDL + 1.6×wLL = 1.2×0.02083 + 1.6×0.03500 = 0.08100 kip/in

Vi = wext×(L/2 − x) = 0.08100×(384 − 18) = 29.646 kip

Mmax = wext×x×(L−x)/2 = 0.08100×18×750/2 = 546.75 kip-in

Unfactored dead-load shear (self-weight + SDL, no load factor applied):

Vd = wDL×(L/2 − x) = 0.06311×366 = 23.097 kip

Substitution into Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

= 0.6×1.0×0.07746×7.50×22.40 + 23.097 + 71.454

= 7.808 + 23.097 + 71.454 = 102.36 kip

Lower bound check:

Vci,min = 5×λ×√f′c×bw×dp = 5×1.0×0.07746×7.50×22.40 = 65.07 kip

Vci = max(102.36, 65.07) = 102.36 kip

Vcw calculation

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

(3.5×0.07746 + 0.3×0.1745)×7.50×22.40 = 54.343 kip

Vp = 2.538 kip

Vcw = 54.343 + 2.538 = 56.881 kip

Concrete Shear Capacity and Stirrup Check

Per Eq. (4), the concrete shear capacity is taken as the lesser of the two failure modes, because the section will crack by whichever mechanism requires the smaller shear force:

Vc = min(Vci, Vcw) = min(102.36, 56.881) = 56.88 kip

→ Vcw governs (web-shear cracking controls)

Design shear capacity:

φVc = 0.75×56.88 = 42.66 kip

Factored shear demand at this section:

Vu = wu×(L/2 − x) = 0.13173×366 = 48.21 kip

Vu = 48.21 kip > φVc = 42.66 kip → Shear reinforcement required beyond minimum

Required steel contribution from Eq. (5):

Vs = Vu/φv − Vc = 48.21/0.75 − 56.88 = 7.40 kip

Stirrup sizing: use #3 closed U-stirrups (2 legs × 0.11 in²/leg = Av = 0.22 in²):

Required Av/s from demand (Eq. 6):

Av/sdemand = Vs/(fy×dp) = 7.40/(60×22.40) = 0.00551 in²/in

Minimum Av/s (from Section 4, Criterion 1 governs):

Av/smin = 0.00726 in²/in > 0.00551 in²/in → minimum governs

Spacing:

sreq = Av/(Av/s) = 0.22/0.00726 = 30.30 in.

Maximum spacing per ACI 318-19 Section 9.7.6.2.2:

smax = min(3h/4, 24 in.) = min(3×28/4, 24) = min(21.0, 24) = 21.0 in.

s = min(req, smax) = min(30.30, 21.0) = 21 in. ← USE

3.2 Section 2: x = 120 in. (10.0 ft)

Prestress at section

yps = 9.590 in.,   e = 10.772 in.

Cracking Moment

fce = Pe/Ac + Pe×e×yb/Ic

= 85.0/487.0 + 85.0×10.772×20.362/34,638.8

= 0.1745 + 0.5383 = 0.7128 ksi

fd = Md×yb/Ic

where Md = wDL×x×(L−x)/2 = 0.06311×120×(768−120)/2 = 2,453.5 kip-in

fd = 2,453.5×20.362/34,638.8 = 1.4423 ksi

fr + fce − fd = 0.4648 + 0.7128 − 1.4423 = −0.2648 ksi < 0

Mcr = 0 (section cracked under dead load alone)

Flexural-Shear Strength (Vci)

From Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

Factored external-load shear and moment (superimposed loads only, excluding self-weight):

Vi = wext×(L/2 − x) = 0.08100×(384 − 120) = 21.384 kip

Unfactored dead-load shear (self-weight + SDL, no load factor applied):

Vd = wDL×(L/2 − x) = 0.06311×264 = 16.660 kip

Substitution into Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

= 0.6×1.0×0.07746×7.50×22.40 + 16.660 + 0

= 7.808 + 16.660 + 0 = 24.47 kip

Lower bound check:

Vci,min = 5×λ×√f′c×bw×dp = 5×1.0×0.07746×7.50×22.40 = 65.07 kip

Vci = max(24.47, 65.07) = 65.07 kip

Vcw calculation

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

(3.5×0.07746 + 0.3×0.1745)×7.50×22.40 = 54.343 kip

Vp = 2.538 kip

Vcw = 54.343 + 2.538 = 56.881 kip

Concrete Shear Capacity and Stirrup Check

Per Eq. (4), the concrete shear capacity is taken as the lesser of the two failure modes, because the section will crack by whichever mechanism requires the smaller shear force:

Vc = min(Vci, Vcw) = min(65.07, 56.881) = 56.88 kip

→ Vcw governs (web-shear cracking controls)

Design shear capacity:

φVc = 0.75×56.88 = 42.66 kip

Factored shear demand at this section:

Vu = wu×(L/2 − x) = 0.13173×264 = 34.78 kip

Vu = 34.78 kip ≤ φVc = 42.66 kip → Minimum stirrups sufficient

Use #3 closed U-stirrups (2 legs × 0.11 in²/leg = Av = 0.22 in²). Spacing governed by minimum Av/s and ACI 318-19 Section 9.7.6.2.2:

smax = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.

s = min(0.22/0.00726, 21.0) = min(30.30, 21.0) = 21 in. ← USE

3.3 Section 3: x = 240 in. (20.0 ft)

Prestress at section

yps = 6.000 in.,   e = 14.362 in.

Cracking Moment

fce = Pe/Ac + Pe×e×yb/Ic

= 85.0/487.0 + 85.0×14.362×20.362/34,638.8

= 0.1745 + 0.7177 = 0.8922 ksi

fd = Md×yb/Ic

where Md = wDL×x×(L−x)/2 = 0.06311×240×(768−240)/2 = 3,997.4 kip-in

fd = 3,997.4×20.362/34,638.8 = 2.3505 ksi

fr + fce − fd = 0.4648 + 0.8922 − 2.3505 = −0.9936 ksi < 0

Mcr = 0 (section cracked under dead load alone)

Flexural-Shear Strength (Vci)

From Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

Factored external-load shear and moment (superimposed loads only, excluding self-weight):

Vi = wext×(L/2 − x) = 0.08100×(384 − 240) = 11.664 kip

Unfactored dead-load shear (self-weight + SDL, no load factor applied):

Vd = wDL×(L/2 − x) = 0.06311×144 = 9.088 kip

Substitution into Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

= 0.6×1.0×0.07746×7.50×22.40 + 9.088 + 0

= 7.808 + 9.088 + 0 = 16.90 kip

Lower bound check:

Vci,min = 5×λ×√f′c×bw×dp = 5×1.0×0.07746×7.50×22.40 = 65.07 kip

Vci = max(16.90, 65.07) = 65.07 kip

Vcw calculation

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

(3.5×0.07746 + 0.3×0.1745)×7.50×22.40 = 54.343 kip

Vp = 0.000 kip

Vcw = 54.343 + 0.000 = 54.343 kip

Concrete Shear Capacity and Stirrup Check

Per Eq. (4), the concrete shear capacity is taken as the lesser of the two failure modes, because the section will crack by whichever mechanism requires the smaller shear force:

Vc = min(Vci, Vcw) = min(65.07, 54.343) = 54.34 kip

→ Vcw governs (web-shear cracking controls)

Design shear capacity:

φVc = 0.75×54.34 = 40.76 kip

Factored shear demand at this section:

Vu = wu×(L/2 − x) = 0.13173×144 = 18.97 kip

Vu = 18.97 kip ≤ φVc = 40.76 kip → Minimum stirrups sufficient

Use #3 closed U-stirrups (2 legs × 0.11 in²/leg = Av = 0.22 in²). Spacing governed by minimum Av/s and ACI 318-19 Section 9.7.6.2.2:

smax = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.

s = min(0.22/0.00726, 21.0) = min(30.30, 21.0) = 21 in. ← USE

3.4 Section 4: x = 384 in. (32.0 ft)

Prestress at section

yps = 6.000 in.,   e = 14.362 in.

Cracking Moment

fce = Pe/Ac + Pe×e×yb/Ic

= 85.0/487.0 + 85.0×14.362×20.362/34,638.8

= 0.1745 + 0.7177 = 0.8922 ksi

fd = Md×yb/Ic

where Md = wDL×x×(L−x)/2 = 0.06311×384×(768−384)/2 = 4,651.1 kip-in

fd = 4,651.1×20.362/34,638.8 = 2.7351 ksi

fr + fce − fd = 0.4648 + 0.8922 − 2.7351 = −1.3781 ksi < 0

Mcr = 0 (section cracked under dead load alone)

Flexural-Shear Strength (Vci)

From Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

Factored external-load shear and moment (superimposed loads only, excluding self-weight):

Vi = wext×(L/2 − x) = 0.08100×(384 − 384) = 0.000 kip

Unfactored dead-load shear (self-weight + SDL, no load factor applied):

Vd = wDL×(L/2 − x) = 0.06311×0 = 0.000 kip

Substitution into Eq. (1):

Vci = 0.6×λ×√f′c×bw×dp + Vd + (Vi/Mmax)×Mcr

= 0.6×1.0×0.07746×7.50×22.40 + 0.000 + 0

= 7.808 + 0.000 + 0 = 7.81 kip

Lower bound check:

Vci,min = 5×λ×√f′c×bw×dp = 5×1.0×0.07746×7.50×22.40 = 65.07 kip

Vci = max(7.81, 65.07) = 65.07 kip

Vcw calculation

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

(3.5×0.07746 + 0.3×0.1745)×7.50×22.40 = 54.343 kip

Vp = 0.000 kip

Vcw = 54.343 + 0.000 = 54.343 kip

Concrete Shear Capacity and Stirrup Check

Per Eq. (4), the concrete shear capacity is taken as the lesser of the two failure modes, because the section will crack by whichever mechanism requires the smaller shear force:

Vc = min(Vci, Vcw) = min(65.07, 54.343) = 54.34 kip

→ Vcw governs (web-shear cracking controls)

Design shear capacity:

φVc = 0.75×54.34 = 40.76 kip

Factored shear demand at this section:

Vu = wu×(L/2 − x) = 0.13173×0 = 0.00 kip

Vu = 0.00 kip ≤ φVc = 40.76 kip → Minimum stirrups sufficient

Use #3 closed U-stirrups (2 legs × 0.11 in²/leg = Av = 0.22 in²). Spacing governed by minimum Av/s and ACI 318-19 Section 9.7.6.2.2:

smax = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.

s = min(0.22/0.00726, 21.0) = min(30.30, 21.0) = 21 in. ← USE

4. Minimum Shear Reinforcement (ACI 318-19 Table 9.6.3.3)

Criterion 1:

Av/s = 0.75×√f′c×bw/fy

= 0.75×0.07746×7.50/60 = 0.00726 in²/in  ← GOVERNS

Criterion 2:

Av/s = (50/1000)×bw/fy

= 0.050×7.50/60 = 0.00625 in²/in

Criterion 3:

Av/s = (Aps×fpu)/(80×fy×dp)×√(dp/bw)

= (0.612×270)/(80×60×22.4)×√(22.4/7.5) = 0.00266 in²/in

Selected bar: #3 closed U-stirrups, Av = 2×0.11 = 0.22 in²

Spacing limits:

Basic: smax = min(3h/4, 24) = min(21.0, 24) = 21.0 in.

Tight (when Vs > 4×√f′c×bw×dp): smax = min(3h/8, 12) = 10.5 in. — not triggered

5. Summary and Design

| Section | x (ft) | Vu (kip) | Vci (kip) | Vcw (kip) | Vc (kip) | φVc (kip) | s (in.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.5 | 48.21 | 102.36 | 56.88 | 56.88 | 42.66 | 21 |
| 2 | 10.0 | 34.78 | 65.07 | 56.88 | 56.88 | 42.66 | 21 |
| 3 | 20.0 | 18.97 | 65.07 | 54.34 | 54.34 | 40.76 | 21 |
| 4 | 32.0 | 0.00 | 65.07 | 54.34 | 54.34 | 40.76 | 21 |

Design Recommendation

Vcw governs at all four design sections. Only Section 1 (x = 1.5 ft) requires stirrups beyond the minimum (Vs = 7.40 kip), but minimum Av/s still governs the spacing. The recommended stirrup design is #3 U-stirrups at 21 in. throughout the span.

Maximum Vs Check

Vs,max = 8×λ×√f′c×bw×dp = 8×1.0×0.07746×7.50×22.40 = 104.1 kip. At Section 1, Vs = 7.40 kip ≤ 104.1 kip. Section size is adequate.

6. Shear Design Figure

Fig. 1 — Shear demand, concrete capacity, required Av/s, and stirrup spacing along the beam span.

ACI Code References

| ACI 318-19 Reference | Description |
| --- | --- |
| Table 22.5.8.2 | Detailed method (Vci/Vcw) |
| Eq. 22.5.8.3.1a | Vci flexural-shear |
| Eq. 22.5.8.3.2 | Vcw web-shear |
| Sec. 22.5.3.2 | Critical section at dp |
| Table 9.6.3.3 | Minimum Av/s |
| Sec. 9.7.6.2.2 | Spacing limits |
| Table 21.2.1 | φ = 0.75 for shear |