CEE 530 PRESTRESSED CONCRETE

Project 1 — Assignment 4: Shear Design

Shear Design of Prestressed Double-T Beam

Spring 2026

Group 3

Hailey Crosson

Reyaneh Kazemian

Leohuns Robert Kambu

Prince Aminu

Chidchanok Pleesudjai (Fen)

Haribhushan Sirigineedi

SIGN CONVENTION: Compression = positive (+)     Tension = negative (−)

UNIT SYSTEM: kip, in. (moments in kip-in., stresses in ksi)

METHOD: Detailed method — Vci / Vcw (ACI 318-19 Table 22.5.8.2)

Part 1 — Given Data

1.1  Beam Geometry

Section type: Double-T (precast, simply supported)

Span: L = (20 + 24 + 20) ft × 12 in./ft = 768 in. (64.0 ft)

Total depth: h = 28.00 in.

Fig. 1 — Beam elevation and support conditions

1.2  Section Properties

Fig. 2 — Section dimensions and vertex numbering

| Property | Value |
| --- | --- |
| Ac | 487.000 in² |
| Ic | 34,638.82 in⁴ |
| yc (from bottom) | 20.362 in. |
| yb | 20.362 in. |
| yt | 7.638 in. |
| bw (sum of stem widths) | 7.50 in. |
| St = Ic/yt | 4,535.1 in³ |
| Sb = Ic/yb | 1,701.1 in³ |

1.3  Material Properties

| Property | Value |
| --- | --- |
| f′c | 6.0 ksi |
| f′ci | 4.8 ksi |
| Ec | 4,700 ksi |
| fpu | 270 ksi |
| fpy | 243 ksi |
| Eps | 28,500 ksi |
| fy (stirrups) | 60 ksi |
| λ | 1.0 (normal weight) |
| φv | 0.75 (ACI Table 21.2.1) |

√(f′c in psi) = √(6000) / 1000 = 0.07746 ksi  (used throughout shear equations)

1.4  Prestress Data

Four 0.5-in. diameter Grade 270 strands, each Aps = 0.153 in².

Aps,total = 4 × 0.153 = 0.612 in²

Pi per strand = 25.0 kip   →   Fi,total = 4 × 25.0 = 100.0 kip

fpi = Pi/Aps = 25.0 / 0.153 = 163.4 ksi

Loss factor: η = 1 − 0.15 = 0.85

Pe,total = η × Fi = 0.85 × 100.0 = 85.0 kip

Tendon layout (y from bottom):

Tendons 1, 3 (straight): y = 6.00 in. (constant)

Tendons 2, 4 (harped): y = 20.36 in. at supports, sloping to y = 6.00 in. at x = 240 in., flat to x = 528 in., then sloping back.

Vertical prestress component Vp (harped tendons, sloped region 0 ≤ x ≤ 240 in.):

tan(α) = (20.36 − 6.00) / 240 = 14.36 / 240 = 0.05983

Pe per harped tendon = 0.153 × 163.4 × 0.85 = 21.25 kip

Vp = 2 × 21.25 × sin(α) = 2 × 21.25 × 0.05972

Vp = 2.538 kip (upward)        [0 in flat zone, x ≥ 240 in.]

Need to add figure here

1.5  Applied Loads

Self-weight:

w_sw = Ac × (150/1728) / 1000 = 487.0 × 0.08681 / 1000

w_sw = 0.04227 kip/in. (0.5073 kip/ft)

Superimposed dead load (2-in. topping, 150 pcf):

w_SDL = 2 × 120 × (150/1728) / 1000

w_SDL = 0.02083 kip/in. (0.2500 kip/ft)

Live load:

w_LL = 0.42 kip/ft / 12 = 0.03500 kip/in. (0.4200 kip/ft)

Total unfactored dead load:

w_DL = w_sw + w_SDL = 0.04227 + 0.02083 = 0.06311 kip/in.

Factored load (ACI 318-19 Sec. 5.3.1b):

wu = 1.2 × w_DL + 1.6 × w_LL

= 1.2 × 0.06311 + 1.6 × 0.03500

= 0.07573 + 0.05600

wu = 0.13173 kip/in. (1.5808 kip/ft)

Part 2 — Shear Design Method

ACI 318-19 requires that the factored shear demand Vu at each section shall not exceed the design shear capacity φVn, where Vn = Vc + Vs. The concrete contribution Vc is taken as the lesser of two failure modes: flexural-shear cracking (Vci) and web-shear cracking (Vcw).

Flexural-shear cracking (Vci) initiates as a flexural crack that subsequently turns into a diagonal shear crack. It governs in regions of high moment (near midspan). A lower bound Vci,min is enforced.

Web-shear cracking (Vcw) initiates as a diagonal crack at the neutral axis before any flexural cracking occurs. It governs near the supports where shear is high and moment is low.

2.1  Key Equations

Eq. (1) — Flexural-shear strength (ACI 318-19 Eq. 22.5.8.3.1a):

Vci = 0.6λ√(f′c) × bw × dp  +  Vd  +  (Vi/Mmax) × Mcr

The rule

Vci≥ Vci,min = 1.7λ√(f′c) × bw × dp

And

Vci< Vci,min = 5.0λ√(f′c) × bw × dp

where Vd = shear from unfactored dead load (SW + SDL);

Vi and Mmax = factored shear and moment from superimposed loads only (1.2×wSDL + 1.6×wLL).

Eq. (2) — Cracking moment (ACI 318-19 Eq. 22.5.8.3.2a):

Mcr = (Ic/yb)(6λ√(f′c) + fce − fd)

where fce = compressive stress at the bottom fiber due to effective prestress alone;

fd = tensile stress at the bottom fiber due to unfactored dead-load moment.

When fd > 6λ√(f′c) + fce, Mcr = 0 and Vci,min governs.

Eq. (3) — Web-shear strength (ACI 318-19 Eq. 22.5.8.3.2):

Vcw = (3.5λ√(f′c) + 0.3fpc) × bw × dp  +  Vp

where fpc = Pe/Ac; Vp = vertical component of harped tendons.

Eq. (4):  Vc = min(Vci, Vcw)

Eq. (5):  Vs,req = max(Vu/φv − Vc, 0)

Eq. (6):  Av/s = Vs / (fy × dp)

2.2  Lower Bound Vci,min (ACI 318-19 Table 22.5.8.2)

ACI 318-19 mandates that Vci shall not be taken less than a lower bound value. The full three-term Vci equation depends on the ratio Vi/Mmax (factored external shear to moment). In regions of high moment and low shear (near midspan), this ratio approaches zero, causing the third term to vanish. Additionally, when dead-load tension exceeds the sum of prestress compression and the modulus of rupture (fd > fr + fce), Mcr ≤ 0, and Vci collapses to an unrealistically low value. The lower bound recognizes that even after flexural cracking, concrete retains shear capacity from aggregate interlock, dowel action of reinforcement, and the uncracked compression zone.

Formula:

ACI 318-19:  Vci,min = 1.7 × λ × √(f′c) × bw × dp

CEE 530:     Vci,min = 1.7 × λ × √(f′c) × bw × dp    ← used in this report

Substitution (CEE 530, coeff = 5.0):

Vci,min = 1.5 × 1.0 × 0.07746 × 7.50 × 22.40

= 5.0 × 13.014

Vci,min = 65.07 kip

Comparison with ACI 318-19 (coeff = 1.7):

Vci,min(ACI) = 1.7 × 1.0 × 0.07746 × 7.50 × 22.40 = 22.12 kip

The CEE 530 coefficient of 5.0 (versus ACI’s 1.7) yields a significantly higher floor. With Vci,min = 65.07 kip exceeding Vcw = 54.34–56.88 kip at all sections, Vcw governs as Vc = min(Vci, Vcw) throughout the beam. For Mcr = 0 sections (x ≥ 10 ft from each support), Vci,min governs over the three-term equation (marked with * in Part 5 summary table).

Part 3 — Detailed Calculations at Section 1: x = 18 in. (1.5 ft)

3.1  Effective Depth dp

dp shall not be less than 0.80h per ACI 318-19 Sec. 22.5.3.2.

Tendon centroid at x = 18 in.:

T1,T3 (straight): y = 6.00 in.

T2,T4 (harped):   y = 20.36 + (6.00−20.36)×(18/240) = 19.28 in.

y_ps = (6.00 + 19.28 + 6.00 + 19.28) / 4 = 12.642 in.

e = yc − y_ps = 20.362 − 12.642 = 7.721 in.

dp = max(h − y_ps, 0.80h)

= max(28.00 − 12.642, 0.80 × 28.00)

= max(15.358, 22.40)

dp = 22.40 in.   [0.80h governs]

3.2  Modulus of Rupture fr

Per ACI 318-19 Table 22.5.8.2, fr for Vci/Mcr uses 6λ√(f′c in psi).

fr = 6 × 1.0 × √(6000) / 1000 = 6 × 77.46 / 1000

fr = 0.4648 ksi

3.3  Prestress Stress at Bottom Fiber fce

fce = compressive stress at bottom fiber due to effective prestress alone (positive = compression).

Formula:  fce = Pe/Ac + Pe × e × yb / Ic

Substitution:

fce = 85.0/487.0 + 85.0 × 7.721 × 20.362 / 34,638.82

= 0.1745 + 13,359.3 / 34,638.82

= 0.1745 + 0.3858

fce = 0.5603 ksi (compression)

3.4  Dead-Load Stress at Bottom Fiber fd

Md = w_DL × x × (L−x) / 2

= 0.06311 × 18 × (768−18) / 2

= 0.06311 × 18 × 750 / 2

Md = 426.0 kip-in.

fd = Md × yb / Ic = 426.0 × 20.362 / 34,638.82

fd = 0.2504 ksi

3.5  Cracking Moment Mcr

fr + fce − fd = 0.4648 + 0.5603 − 0.2504 = 0.7747 ksi  > 0  ✔

Mcr = (Ic/yb) × (fr + fce − fd)

= 1,701.1 × 0.7747

Mcr = 1,317.8 kip-in. (109.8 kip-ft)

3.6  Flexural-Shear Strength Vci

Load terms at x = 18 in.:

w_ext = 1.2 × w_SDL + 1.6 × w_LL = 1.2×0.02083 + 1.6×0.03500 = 0.08100 kip/in.

Vi   = w_ext × (L/2 − x) = 0.08100 × (384 − 18) = 0.08100 × 366 = 29.646 kip

Mmax = w_ext × x × (L−x)/2 = 0.08100 × 18 × 750/2 = 546.75 kip-in.

Vd   = w_DL × (L/2 − x) = 0.06311 × 366 = 23.097 kip

Three-term Vci from Eq. (1):

Term 1: 0.6 × 1.0 × 0.07746 × 7.50 × 22.40 = 7.808 kip

Term 2: Vd = 23.097 kip

Term 3: (Vi/Mmax) × Mcr = (29.646/546.75) × 1,317.8 = 71.454 kip

Sum:    7.808 + 23.097 + 71.454 = 102.36 kip

Lower bound check:

Vci,min = 5 × 1.0 × 0.07746 × 7.50 × 22.40 = 65.07 kip

Vci = max(102.36, 65.07) = 102.36 kip  ≥  Vci,min = 65.07 kip  →  PASS

Vci = 102.36 kip

3.7  Web-Shear Strength Vcw

Centroidal prestress stress fpc:

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

Vertical prestress component Vp (x = 18 in., in sloped region):

Vp = 2.538 kip (upward)

Vcw from Eq. (3):

(3.5 × 0.07746 + 0.3 × 0.1745) × 7.50 × 22.40

= (0.27111 + 0.05235) × 168.00 = 54.343 kip

Vcw = 54.343 + 2.538

Vcw = 56.881 kip

3.8  Concrete Shear Capacity Vc

Vc = min(Vci, Vcw) = min(102.36, 56.881)

Vc = 56.88 kip   → Vcw GOVERNS (web-shear cracking controls)

φVc = 0.75 × 56.88 = 42.66 kip

3.9  Shear Demand vs. Capacity

Vu = wu × (L/2 − x) = 0.13173 × (384 − 18) = 0.13173 × 366

Vu = 48.21 kip

Code check (ACI 318-19 Sec. 22.5.1.1):

Vu = 48.21 kip  >  φVc = 42.66 kip  →  FAIL

STATUS: Shear reinforcement required beyond minimum

3.10  Required Steel Contribution Vs

Vs,req = max(Vu/φv − Vc, 0)

= max(48.21/0.75 − 56.88, 0)

= max(64.28 − 56.88, 0)

Vs,req = 7.40 kip

Maximum Vs check (ACI 318-19 Sec. 22.5.1.2):

Vs,max = 8 × λ × √(f′c) × bw × dp = 8 × 0.07746 × 7.50 × 22.40 = 104.1 kip

Vs,req = 7.40 kip  ≤  Vs,max = 104.1 kip  (section size adequate)  →  PASS

3.11  Stirrup Design

Selected bar: #3 closed U-stirrups (2 legs × 0.11 in.²/leg = Av = 0.22 in.²)

Av/s from demand (Eq. 6):

Av/s_demand = Vs/(fy × dp) = 7.40/(60 × 22.40) = 7.40/1344

Av/s_demand = 0.00551 in.²/in.

Av/s minimum (from Part 4, Criterion 1 governs): 0.00726 in.²/in.

Av/s_min = 0.00726 > 0.00551  →  minimum governs

Spacing:

s_req = Av / (Av/s) = 0.22 / 0.00726 = 30.30 in.

s_max = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.  [ACI Sec. 9.7.6.2.2]

s = min(30.30, 21.0)

s = 21 in.   ← USE

Verify capacity:

Vs,prov = Av × fy × dp / s = 0.22 × 60 × 22.40 / 21 = 14.08 kip

φ(Vc + Vs) = 0.75 × (56.88 + 14.08) = 0.75 × 70.96 = 53.22 kip

φ(Vc + Vs) = 53.22 kip  ≥  Vu = 48.21 kip  →  PASS

Part 4 — Minimum Shear Reinforcement (ACI 318-19 Table 9.6.3.3)

Three criteria shall be evaluated; the largest governs.

Criterion 1 (Eq. 9.6.3.3a):

Av/s = 0.75 × √(f′c in psi) × bw / fy

= 0.75 × 0.07746 × 7.50 / 60

Av/s_min1 = 0.00726 in.²/in.    ← GOVERNS

Criterion 2 (Eq. 9.6.3.3b):

Av/s = (50/1000) × bw / fy = 0.050 × 7.50 / 60

Av/s_min2 = 0.00625 in.²/in.

Criterion 3 (Eq. 9.6.3.3c):

Av/s = (Aps × fpu)/(80 × fy × dp) × √(dp/bw)

= (0.612 × 270)/(80 × 60 × 22.40) × √(22.40/7.50)

= 165.24/107,520 × 1.728

Av/s_min3 = 0.00266 in.²/in.

Governing: Av/s_min = max(0.00726, 0.00625, 0.00266) = 0.00726 in.²/in. (Criterion 1)

Stirrup spacing limits (ACI 318-19 Sec. 9.7.6.2.2):

Basic:  s_max = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.

Tight trigger: Vs > 4√(f′c) × bw × dp = 52.06 kip

Vs,req = 7.40 kip ≤ 52.06 kip  →  basic limit applies

Part 5 — Summary at All Design Sections

Values computed at four design sections. bw = 7.50 in., dp = 22.40 in. (0.8h governs throughout), φv = 0.75.

| Section | x (ft) | Vu (kip) | Vci (kip) | Vcw (kip) | Vc (kip) | φVc (kip) | Vs (kip) | Av/s | s (in.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.5 | 48.21 | 102.36 | 56.88 | 56.88 | 42.66 | 7.40 | 0.00726 | 21 |
| 2 | 10.0 | 34.78 | 65.07* | 56.88 | 56.88 | 42.66 | 0 | 0.00726 | 21 |
| 3 | 20.0 | 18.97 | 65.07* | 54.34 | 54.34 | 40.76 | 0 | 0.00726 | 21 |
| 4 | 32.0 | 0.00 | 65.07* | 54.34 | 54.34 | 40.76 | 0 | 0.00726 | 21 |

(*) Mcr = 0 at these sections (DL tension exceeds prestress + rupture); Vci = Vci,min = 65.07 kip.

Key observation: Vcw governs at all four design sections. Only Section 1 (x = 1.5 ft) requires stirrups beyond the minimum (Vs = 7.40 kip), but the minimum Av/s still governs the spacing.

Part 6 — Design Recommendation

Based on the analysis at all design sections:

Recommended stirrup design: #3 U-stirrups @ 21 in. throughout the span.

Vcw governs everywhere. Even at Section 1, where Vs = 7.40 kip is required beyond the concrete capacity, the minimum Av/s (0.00726 in.²/in.) governs over the demand Av/s (0.00551 in.²/in.). The resulting spacing of 30.30 in. is then reduced to 21 in. by the ACI spacing limit of 3h/4 = 21 in. A uniform spacing of 21 in. satisfies all locations.

Part 7 — Final Summary

| Check | Demand | Capacity | Status |
| --- | --- | --- | --- |
| dp ≥ 0.80h | 22.40 in. | 22.40 in. | OK (=) |
| Vu at Section 1 | 48.21 kip | — | — |
| φVc at Section 1 | — | 42.66 kip | — |
| Stirrups required? | Vu > φVc | — | YES |
| Vs,req (Section 1) | 7.40 kip | — | — |
| Vs ≤ Vs,max (section size) | 7.40 kip | 104.1 kip | PASS |
| φ(Vc + Vs) ≥ Vu | 48.21 kip | 53.22 kip | PASS |
| Av/s_min (Crit. 1 governs) | 0.00726 | — | — |
| s at Section 1 | 21 in. | 21 in. | USE |

ACI Code References

| ACI 318-19 Reference | Description |
| --- | --- |
| Table 22.5.8.2 | Detailed method (Vci / Vcw) |
| Eq. 22.5.8.3.1a | Vci flexural-shear |
| Eq. 22.5.8.3.2a | Mcr cracking moment |
| Eq. 22.5.8.3.2 | Vcw web-shear |
| Sec. 22.5.3.2 | Critical section at dp from support |
| Table 9.6.3.3 | Minimum Av/s (3 criteria) |
| Sec. 9.7.6.2.2 | Stirrup spacing limits (3h/4, 24 in.) |
| Table 21.2.1 | φ = 0.75 for shear |
| Sec. 5.3.1b | Factored load combination (1.2D + 1.6L) |
| Sec. 22.5.1.2 | Maximum Vs limit (8√f′c × bw × dp) |