CEE 530 PRESTRESSED CONCRETE

Project 1 — Assignment 4: Shear Design

Shear Design of Prestressed Double-T Beam

ACI 318-19 / Naaman Section 5.7 — Empirical Method (Vci / Vcw)

Spring 2026   |   Due: 3/23/2026

Group 3

Hailey Crosson

Reyaneh Kazemian

Leohuns Robert Kambu

Prince Aminu

Chidchanok Pleesudjai (Fen)

Haribhushan Sirigineedi

Code Edition: CEE 530 (ACI 318-19 shear provisions)

SIGN CONVENTION: Compression = positive (+)     Tension = negative (−)

UNIT SYSTEM: kip, in. (moments in kip-in., stresses in ksi)

METHOD: Detailed method — Vci / Vcw (ACI 318-19 Table 22.5.8.2)

Part 1 — Given Data

1.1  Beam Geometry

Section type: Double-T (precast, simply supported)

Span: L = (20 + 24 + 20) ft × 12 in./ft = 768 in. (64.0 ft)

Total depth: h = 28.00 in.

Fig. 1 — Beam elevation and support conditions

1.2  Section Properties

Fig. 2 — Section dimensions and vertex numbering

| Property | Value |
| --- | --- |
| Ac | 487.000 in² |
| Ic | 34,638.82 in⁴ |
| yc (from bottom) | 20.362 in. |
| yb | 20.362 in. |
| yt | 7.638 in. |
| bw (sum of stem widths) | 7.50 in. |
| St = Ic/yt | 4,535.1 in³ |
| Sb = Ic/yb | 1,701.1 in³ |

1.3  Material Properties

| Property | Value |
| --- | --- |
| f′c | 6.0 ksi |
| f′ci | 4.8 ksi |
| Ec | 4,700 ksi |
| fpu | 270 ksi |
| fpy | 243 ksi |
| Eps | 28,500 ksi |
| fy (stirrups) | 60 ksi |
| λ | 1.0 (normal weight) |
| φv | 0.75 (ACI Table 21.2.1) |

√(f′c in psi) = √(6000) / 1000 = 0.07746 ksi  (used throughout shear equations)

1.4  Prestress Data

Four 0.5-in. diameter Grade 270 strands, each Aps = 0.153 in².

Aps,total = 4 × 0.153 = 0.612 in²

Pi per strand = 25.0 kip   →   Fi,total = 4 × 25.0 = 100.0 kip

fpi = Pi/Aps = 25.0 / 0.153 = 163.4 ksi

Loss factor: η = 1 − 0.15 = 0.85

Pe,total = η × Fi = 0.85 × 100.0 = 85.0 kip

Tendon layout (y from bottom):

Tendons 1, 3 (straight): y = 6.00 in. (constant)

Tendons 2, 4 (harped): y = 20.36 in. at supports, sloping to y = 6.00 in. at x = 240 in., flat to x = 528 in., then sloping back.

Vertical prestress component Vp (harped tendons, sloped region 0 ≤ x ≤ 240 in.):

tan(α) = (20.36 − 6.00) / 240 = 14.36 / 240 = 0.05983

Pe per harped tendon = 0.153 × 163.4 × 0.85 = 21.25 kip

Vp = 2 × 21.25 × sin(α) = 2 × 21.25 × 0.05972

Vp = 2.538 kip (upward)        [0 in flat zone, x ≥ 240 in.]

1.5  Applied Loads

Self-weight:

w_sw = Ac × (150/1728) / 1000 = 487.0 × 0.08681 / 1000

w_sw = 0.04227 kip/in. (0.5073 kip/ft)

Superimposed dead load (2-in. topping, 150 pcf):

w_SDL = 2 × 120 × (150/1728) / 1000

w_SDL = 0.02083 kip/in. (0.2500 kip/ft)

Live load:

w_LL = 0.42 kip/ft / 12 = 0.03500 kip/in. (0.4200 kip/ft)

Total unfactored dead load:

w_DL = w_sw + w_SDL = 0.04227 + 0.02083 = 0.06311 kip/in.

Factored load (ACI 318-19 Sec. 5.3.1b):

wu = 1.2 × w_DL + 1.6 × w_LL

= 1.2 × 0.06311 + 1.6 × 0.03500

= 0.07573 + 0.05600

wu = 0.13173 kip/in. (1.5808 kip/ft)

Part 2 — Shear Design Method

ACI 318-19 requires that the factored shear demand Vu at each section shall not exceed the design shear capacity φVn, where Vn = Vc + Vs. The concrete contribution Vc is taken as the lesser of two failure modes: flexural-shear cracking (Vci) and web-shear cracking (Vcw).

Flexural-shear cracking (Vci) initiates as a flexural crack that subsequently turns into a diagonal shear crack. It governs in regions of high moment (near midspan). A lower bound Vci,min is enforced.

Web-shear cracking (Vcw) initiates as a diagonal crack at the neutral axis before any flexural cracking occurs. It governs near the supports where shear is high and moment is low.

2.1  Key Equations

Eq. (1) — Flexural-shear strength (ACI 318-19 Eq. 22.5.8.3.1a):

Vci = 0.6λ√(f′c) × bw × dp  +  Vd  +  (Vi/Mmax) × Mcr

≥ Vci,min = 1.7λ√(f′c) × bw × dp      [ACI 318-19 hard floor]

≤ Vci,max = 5.0λ√(f′c) × bw × dp      [CEE 530 convention — governs]

where Vd = shear from unfactored dead load (SW + SDL); Vi and Mmax = factored shear and moment from superimposed loads only (1.2×wSDL + 1.6×wLL).

Eq. (2) — Cracking moment (ACI 318-19 Eq. 22.5.8.3.2a):

Mcr = (Ic/yb)(6λ√(f′c) + fce − fd)

where fce = compressive stress at the bottom fiber due to effective prestress alone; fd = tensile stress at the bottom fiber due to unfactored dead-load moment. When fd > 6λ√(f′c) + fce, Mcr = 0 and Vci,min governs.

Eq. (3) — Web-shear strength (ACI 318-19 Eq. 22.5.8.3.2):

Vcw = (3.5λ√(f′c) + 0.3fpc) × bw × dp  +  Vp

where fpc = Pe/Ac; Vp = vertical component of harped tendons.

Eq. (4):  Vc = min(Vci, Vcw)

Eq. (5):  Vs,req = max(Vu/φv − Vc, 0)

Eq. (6):  Av/s = Vs / (fy × dp)

2.2  Bounds on Vci

Lower bound (ACI 318-19 hard floor):  Prevents Vci from dropping unrealistically low near midspan where Vi → 0 or Mcr = 0:

Vci ≥ Vci,min = 1.7λ√(f′c) × bw × dp

Without this floor, Vci would reduce to just Vd + 0.6λ√(f′c)×bw×dp in regions where DL tension exceeds prestress + rupture (Mcr = 0) — an unconservatively small value.

Upper bound (CEE 530, Prof. Mobasher):  Caps Vci near supports where Vi/Mmax → ∞ would inflate the three-term formula:

Vci ≤ Vci,max = 5.0λ√(f′c) × bw × dp

The 5.0 coefficient equals the upper cap on the simplified method Vc (ACI 318-19 Sec. 22.5.8.1). Using it as the Vci ceiling in the detailed method ensures that Vci can never exceed the simplified-method maximum — a course convention.

For this beam (bw = 7.50 in., dp = 22.40 in., f′c = 6.0 ksi):

Vci,min (ACI 318-19) = 1.7 × 1.0 × 0.07746 × 7.50 × 22.40 = 22.12 kip  [floor]

Vci,max (CEE 530)    = 5.0 × 1.0 × 0.07746 × 7.50 × 22.40 = 65.07 kip  [cap]

Part 3 — Detailed Calculations at Section 1: x = 18 in. (1.5 ft)

3.1  Effective Depth dp

dp shall not be less than 0.80h per ACI 318-19 Sec. 22.5.3.2.

Tendon centroid at x = 18 in.:

T1,T3 (straight): y = 6.00 in.

T2,T4 (harped):   y = 20.36 + (6.00−20.36)×(18/240) = 19.28 in.

y_ps = (6.00 + 19.28 + 6.00 + 19.28) / 4 = 12.642 in.

e = yc − y_ps = 20.362 − 12.642 = 7.721 in.

dp = max(h − y_ps, 0.80h)

= max(28.00 − 12.642, 0.80 × 28.00)

= max(15.358, 22.40)

dp = 22.40 in.   [0.80h governs]

3.2  Modulus of Rupture fr

Per ACI 318-19 Table 22.5.8.2, fr for Vci/Mcr uses 6λ√(f′c in psi).

fr = 6 × 1.0 × √(6000) / 1000 = 6 × 77.46 / 1000

fr = 0.4648 ksi

3.3  Prestress Stress at Bottom Fiber fce

fce = compressive stress at bottom fiber due to effective prestress alone (positive = compression).

Formula:  fce = Pe/Ac + Pe × e × yb / Ic

Substitution:

fce = 85.0/487.0 + 85.0 × 7.721 × 20.362 / 34,638.82

= 0.1745 + 13,359.3 / 34,638.82

= 0.1745 + 0.3858

fce = 0.5603 ksi (compression)

3.4  Dead-Load Stress at Bottom Fiber fd

Md = w_DL × x × (L−x) / 2

= 0.06311 × 18 × (768−18) / 2

= 0.06311 × 18 × 750 / 2

Md = 426.0 kip-in.

fd = Md × yb / Ic = 426.0 × 20.362 / 34,638.82

fd = 0.2504 ksi

3.5  Cracking Moment Mcr

fr + fce − fd = 0.4648 + 0.5603 − 0.2504 = 0.7747 ksi  > 0  ✔

Mcr = (Ic/yb) × (fr + fce − fd)

= 1,701.1 × 0.7747

Mcr = 1,317.8 kip-in. (109.8 kip-ft)

3.6  Flexural-Shear Strength Vci

Load terms at x = 18 in.:

w_ext = 1.2 × w_SDL + 1.6 × w_LL = 1.2×0.02083 + 1.6×0.03500 = 0.08100 kip/in.

Vi   = w_ext × (L/2 − x) = 0.08100 × (384 − 18) = 0.08100 × 366 = 29.646 kip

Mmax = w_ext × x × (L−x)/2 = 0.08100 × 18 × 750/2 = 546.75 kip-in.

Vd   = w_DL × (L/2 − x) = 0.06311 × 366 = 23.097 kip

Three-term Vci from Eq. (1):

Term 1: 0.6 × 1.0 × 0.07746 × 7.50 × 22.40 = 7.808 kip

Term 2: Vd = 23.097 kip

Term 3: (Vi/Mmax) × Mcr = (29.646/546.75) × 1,317.8 = 71.454 kip

Sum:    7.808 + 23.097 + 71.454 = 102.36 kip

Bounds check:

Vci,min (ACI floor) = 1.7 × 1.0 × 0.07746 × 7.50 × 22.40 = 22.12 kip

Vci,max (CEE cap)   = 5.0 × 1.0 × 0.07746 × 7.50 × 22.40 = 65.07 kip

Vci (computed)      = 102.36 kip

Floor:  102.36 kip  ≥  22.12 kip  →  PASS

Cap:    102.36 kip  >  65.07 kip  →  capped to Vci,max  →  FAIL

Vci = min(102.36, 65.07) = 65.07 kip

3.7  Web-Shear Strength Vcw

Centroidal prestress stress fpc:

fpc = Pe/Ac = 85.0/487.0 = 0.1745 ksi

Vertical prestress component Vp (x = 18 in., in sloped region):

Vp = 2.538 kip (upward)

Vcw from Eq. (3):

(3.5 × 0.07746 + 0.3 × 0.1745) × 7.50 × 22.40

= (0.27111 + 0.05235) × 168.00 = 54.343 kip

Vcw = 54.343 + 2.538

Vcw = 56.881 kip

3.8  Concrete Shear Capacity Vc

Vc = min(Vci, Vcw) = min(65.07, 56.881)

Vc = 56.88 kip   → Vcw GOVERNS (web-shear cracking controls)

φVc = 0.75 × 56.88 = 42.66 kip

3.9  Shear Demand vs. Capacity

Vu = wu × (L/2 − x) = 0.13173 × (384 − 18) = 0.13173 × 366

Vu = 48.21 kip

Code check (ACI 318-19 Sec. 22.5.1.1):

Vu = 48.21 kip  >  φVc = 42.66 kip  →  FAIL

STATUS: Shear reinforcement required beyond minimum

3.10  Required Steel Contribution Vs

Vs,req = max(Vu/φv − Vc, 0)

= max(48.21/0.75 − 56.88, 0)

= max(64.28 − 56.88, 0)

Vs,req = 7.40 kip

Maximum Vs check (ACI 318-19 Sec. 22.5.1.2):

Vs,max = 8 × λ × √(f′c) × bw × dp = 8 × 0.07746 × 7.50 × 22.40 = 104.1 kip

Vs,req = 7.40 kip  ≤  Vs,max = 104.1 kip  (section size adequate)  →  PASS

3.11  Stirrup Design

Selected bar: #3 closed U-stirrups (2 legs × 0.11 in.²/leg = Av = 0.22 in.²)

Av/s from demand (Eq. 6):

Av/s_demand = Vs/(fy × dp) = 7.40/(60 × 22.40) = 7.40/1344

Av/s_demand = 0.00551 in.²/in.

Av/s minimum (from Part 4, Criterion 1 governs): 0.00726 in.²/in.

Av/s_min = 0.00726 > 0.00551  →  minimum governs

Spacing:

s_req = Av / (Av/s) = 0.22 / 0.00726 = 30.30 in.

s_max = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.  [ACI Sec. 9.7.6.2.2]

s = min(30.30, 21.0)

s = 21 in.   ← USE

Verify capacity:

Vs,prov = Av × fy × dp / s = 0.22 × 60 × 22.40 / 21 = 14.08 kip

φ(Vc + Vs) = 0.75 × (56.88 + 14.08) = 0.75 × 70.96 = 53.22 kip

φ(Vc + Vs) = 53.22 kip  ≥  Vu = 48.21 kip  →  PASS

Part 4 — Minimum Shear Reinforcement (ACI 318-19 Table 9.6.3.3)

Three criteria shall be evaluated; the largest governs.

Criterion 1 (Eq. 9.6.3.3a):

Av/s = 0.75 × √(f′c in psi) × bw / fy

= 0.75 × 0.07746 × 7.50 / 60

Av/s_min1 = 0.00726 in.²/in.    ← GOVERNS

Criterion 2 (Eq. 9.6.3.3b):

Av/s = (50/1000) × bw / fy = 0.050 × 7.50 / 60

Av/s_min2 = 0.00625 in.²/in.

Criterion 3 (Eq. 9.6.3.3c):

Av/s = (Aps × fpu)/(80 × fy × dp) × √(dp/bw)

= (0.612 × 270)/(80 × 60 × 22.40) × √(22.40/7.50)

= 165.24/107,520 × 1.728

Av/s_min3 = 0.00266 in.²/in.

Governing: Av/s_min = max(0.00726, 0.00625, 0.00266) = 0.00726 in.²/in. (Criterion 1)

Stirrup spacing limits (ACI 318-19 Sec. 9.7.6.2.2):

Basic:  s_max = min(3h/4, 24 in.) = min(21.0, 24) = 21.0 in.

Tight trigger: Vs > 4√(f′c) × bw × dp = 52.06 kip

Vs,req = 7.40 kip ≤ 52.06 kip  →  basic limit applies

Part 5 — Summary at All Design Sections

Values computed at four design sections. bw = 7.50 in., dp = 22.40 in. (0.8h governs throughout), φv = 0.75.

| Section | x (ft) | Vu (kip) | Vci (kip) | Vcw (kip) | Vc (kip) | φVc (kip) | Vs (kip) | Av/s | s (in.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.5 | 48.21 | 65.07† | 56.88 | 56.88 | 42.66 | 7.40 | 0.00726 | 21 |
| 2 | 10.0 | 34.78 | 24.47* | 56.88 | 24.47 | 18.35 | 21.90 | 0.01630 | 13 |
| 3 | 20.0 | 18.97 | 22.12* | 54.34 | 22.12 | 16.59 | 3.17 | 0.00726 | 21 |
| 4 | 32.0 | 0.00 | 22.12* | 54.34 | 22.12 | 16.59 | 0 | 0.00726 | 21 |

(*) Mcr = 0 at these sections; Vci governed by ACI floor (1.7λ√f′c×bw×dp = 22.12 kip) or slightly above.

(†) Vci three-term = 102.36 kip, capped to Vci,max = 5.0λ√f′c×bw×dp = 65.07 kip (CEE 530).

Key observation: Vcw governs at Section 1 (near support). At Sections 2–4, Vci governs because Mcr = 0 drops the three-term Vci to the ACI floor (22.12 kip), which is well below Vcw. Section 2 requires significant stirrup reinforcement (Vs = 21.90 kip, s = 13 in.).

Part 6 — Design Recommendation

Based on the analysis at all design sections:

Recommended stirrup design: #3 U-stirrups @ 13 in. in the support region (0–10 ft), transitioning to 21 in. elsewhere.

With the CEE 530 two-tier bounds (floor = 1.7, cap = 5.0), Vci governs at Sections 2–4 because Mcr = 0 drops the three-term Vci to the ACI floor (22.12 kip). Only at Section 1 does Vcw govern (56.88 kip). The critical section is Section 2 (x = 10 ft), where Vs = 21.90 kip requires Av/s = 0.01630 in.²/in., yielding s = 13 in. Beyond the drape point (x ≥ 20 ft), minimum Av/s governs and the spacing limit of 3h/4 = 21 in. controls.

Part 7 — Shear Design Figure

Fig. 3 — (a) Shear demand vs. concrete capacity; (b) Required Vs; (c) Required Av/s; (d) Stirrup spacing along span.

Part 8 — Final Summary

| Check | Demand | Capacity | Status |
| --- | --- | --- | --- |
| dp ≥ 0.80h | 22.40 in. | 22.40 in. | OK (=) |
| Vu at Section 1 | 48.21 kip | — | — |
| φVc at Section 1 | — | 42.66 kip | — |
| Stirrups required? | Vu > φVc | — | YES |
| Vs,req (Section 1) | 7.40 kip | — | — |
| Vs ≤ Vs,max (section size) | 7.40 kip | 104.1 kip | PASS |
| φ(Vc + Vs) ≥ Vu | 48.21 kip | 53.22 kip | PASS |
| Av/s_min (Crit. 1 governs) | 0.00726 | — | — |
| s at Section 1 | 21 in. | 21 in. | USE |

ACI Code References

| ACI 318-19 Reference | Description |
| --- | --- |
| Table 22.5.8.2 | Detailed method (Vci / Vcw) |
| Eq. 22.5.8.3.1a | Vci flexural-shear (floor: 1.7λ√f′c×bw×dp; CEE 530 uses 5.0) |
| Eq. 22.5.8.3.2a | Mcr cracking moment |
| Eq. 22.5.8.3.2 | Vcw web-shear |
| Sec. 22.5.3.2 | Critical section at dp from support |
| Table 9.6.3.3 | Minimum Av/s (3 criteria) |
| Sec. 9.7.6.2.2 | Stirrup spacing limits (3h/4, 24 in.) |
| Table 21.2.1 | φ = 0.75 for shear |
| Sec. 5.3.1b | Factored load combination (1.2D + 1.6L) |
| Sec. 22.5.1.2 | Maximum Vs limit (8√f′c × bw × dp) |