Project 1

Precast Prestressed Beam

Assignment 2

CEE 530

Prestressed Concrete

Due 3/2/2026

Group 3

Hailey Crosson

Reyaneh Kazemian

Leohuns Robert Kambu

Prince Aminu

Chidchanok Pleesudjai (Fen)

Haribhushan Sirigineedi

Project 1 - Part 2 - Feasibility Chart

Problem Statement:

(1) Draw the feasibility domain at 0′, 10′, 20′ and 32′ from the left support.

(2) Compute the required prestressing force, assuming 15% losses.

Given

## Material Properties and Allowable Stresses

f′c = 6 ksi, f′ci = 4.8 ksi, fpu = 270 ksi

concrete unit weight = 150 lb/ft³ =  0.0868 lb/in³

# Section Properties

Figure 1. Section dimension

Figure 2. Section dimension at the support

# Calculating Stresses (From Project 1-Part 1)

## Eccentricity

Equation for the eccentricity of draped cable from 0 to 20 ft: e1 = (14.36/20)x

@ 0 ft: 		e1 = 0 in,			 e2 = 14.36 in

@ 10 ft: 		e1 = 7.18 in, 			 e2 = 14.36 in

@ 20 ft: 		e1 = 14.36 in, 		 e2 = 14.36 in

@ 32 ft (midspan): e1 = 14.36 in, 		 e2 = 14.36 in

## Moment Due to Self-Weight and Live Load (From Part 1)

- Dead load (self-weight) = 0.0868 lb/in³ × 487 in² = 42.27 lb/in = 507 lb/ft

- Sustained dead load (2″ concrete topping)

= 0.0868 × 2 × 120 = 20.83 lb/in = 250 lb/ft

- Live load = 420 lb/ft

Moment equation: M(x) = 0.5wLx − 0.5wx²

## Material Properties and Allowable Stresses

f′c = 6 ksi, f′ci = 4.8 ksi, fpu = 270 ksi

Allowable stress

transfer:

fc = 0.6 f′ci = 0.6 × 4.8 = −2.88 ksi (−2,880 psi)

ft = 3√(f′ci) = 3√4800 = +207.85 psi (+0.208 ksi)

service:

fc = 0.6 f′c = 0.6 × 6 = −3.6 ksi (−3,600 psi)

ft = 6√(f′c) = 6√6000 = +465 psi (+0.465 ksi)

Calculate the moment at each location condition

Transfer

Service

Stress at bottom and top

Transfer stress

Service Stress

Calculating the constant value

yt, yb = distances of upper and lower edges, respectively, from CGC

yt =−(h − yg) = −7.64 in

yb = (yg) = 20.36 in

kt, kb = distances of upper and lower kern points, respectively, from CGC

St := MOI / yt = −4535.08 in³

Sb := MOI / yb = 1701.15 in³

kt := Sb / A = 3.49 in

kb := St / A = −9.31 in

Section 1, @ 0 ft from the left support, Lx = 0

Moment min

Moment max

Transfer stress

Service Stress

At 0 ft: 		e1 = 0 in and  e2 = 14.36 in

Therefore, the

From Figure 3, the possible range of 1/P is

At Lx = 0,

e_eff = -2.18in

Figure 3

Section 2, @ 10 ft from the left support, Lx = 10ft

Moment min

Moment max

Transfer stress

Service Stress

At 0 ft: 		e1 = 7.18 in and  e2 = 14.36 in

Therefore, the

From Figure 4, the possible range of 1/P is

At Lx = 10,

e_eff = -10.77in

Figure4

Section 3, @ 20 ft from the left support, Lx = 20ft

Moment min

Moment max

Transfer stress

Service Stress

At 0 ft: 		e1 = 14.36 in and  e2 = 14.36 in

Therefore, the

From Figure 5, the possible range of 1/P is

At Lx = 20,

e_eff = -10.77in

e_eff = -14.36in

Figure 5

Section 4, @ 32 ft from the left support, Lx = 32ft

Moment min

Moment max

Transfer stress

Service Stress

At 0 ft: 		e1 = 14.36 in and  e2 = 14.36 in

Therefore, the

From Figure 6, the possible range of 1/P is

At Lx = 32,

e_eff = -10.77in

e_eff = -14.36in

Figure 6

Conclusion

At Lx = 0,

At Lx = 10,

At Lx = 20,

At Lx = 32,

From the feasibility analysis at the four locations, the maximum value of 1P that governs all criteria is:

1Pmax=0.0023

Therefore, the maximum allowable prestressing force is:

Pmax=10.0023=434.78 kip